Details

| Author | Jagannath Narvekar |
| --- | --- |
| Owner | Parveen Jindgar |
| File Name & Location |  |

Version History

| Version | Issue Date | Author | Reason for Change |
| --- | --- | --- | --- |
| 1.0 | 05/02/2022 | Jagannath Narvekar | Initial document |

Document Approvals

| Department | Name | Issue | Date of approval |
| --- | --- | --- | --- |
| Packet Network Engineering | Parveen Jindgar |  |  |

Disclosure / Confidentiality

Highly Confidential, not for distribution

Distribution within Colt

Distribution permitted external to Colt

# This document is valid for MX240/MX480/MX960 devices.

## Prerequisites:

Standard prerequisites for software upgrade:

- Local console accesses of device need to be confirmed prior starting upgrade.

- Back up configuration and capture snapshot of current status of interface, protocols, routing engine, hardware and alarms (if any).

- Telnet/SSH/Console sessions must be captured for entire up-gradation process.

- Check and make sure to have enough storage

## Backup Procedure

Note: All sessions must be logged for commands and command outputs. These logs should be included in the post mortem.

- Save running-configuration to two places (on device locally and NMS)

Juniper machines:

user@DUT> configure private

user@DUT# save filename

user@DUT#run file list --- check if configuration is saved on router

Then use following command to scp the file from router to the NMS server.

nms :~$ scp <DUT device>:/var/tmp/filename ./

Example:

jnarveka@CTEFFPRPER02-re0> configure private

warning: uncommitted changes will be discarded on exit

Entering configuration mode

jnarveka@CTEFFPRPER02-re0# save EFFPER02_10FEB2023

Wrote 40575 lines of configuration to 'EFFPER02_10FEB2023'

jnarveka@CTEFFPRPER02-re0# run file list

var/home/tier3/:

EFFPER02_10FEB2023

jnarveka@nms4:~$ scp effper02.prizmnet:/var/home/tier3/EFFPER02_10FEB2023 ./

Access to this network/system is limited to users authorised by Colt Technology Services Group for its legitimate business purposes. Unauthorised access, use, installation of files or modification of any data is prohibited. Use of this system/network will be audited.

Password:

EFFPER02_10FEB2023   100% |**************************************************************************************************************************************|  1436 KB    00:11

jnarveka@nms4:~$ ls -l

total 2567

-rw-r--r--   1 jnarveka staff    1471111 Feb 10 10:04 EFFPER02_10FEB2023

- For all the routers listed for Upgrade, execute the following show commands and save the information in a text or word document. This information will be used later to check routing.

Make sure to take these outputs pre and post upgrade.

show version invoke-on all-routing-engines | no-more

show system uptime | no-more

show ntp associations no-resolve | no-more

show chassis hardware | no-more

show chassis fpc detail | no-more

show chassis alarms | no-more

show system alarms | no-more

show chassis routing-engine | no-more

show chassis environment | no-more

show system resource-monitor fpc | no-more

show krt table | match "kernel-id:  -" |no-more

show system processes extensive | match rpd |no-more

show interface terse | match "up    up" | no-more

show interface terse | no-more

show interfaces descriptions | no-more

show oam ethernet connectivity-fault-management interfaces extensive | no-more

show arp no-resolve | no-more

show vrrp summary | no-more

show lldp neighbors | no-more

show bfd session | no-more

show mpls interface | no-more

show mpls lsp | no-more

show mpls lsp p2mp | no-more

show bgp summary | no-more

show bgp neighbor | no-more

show isis adjacency extensive | no-more

show route summary | no-more

show route table inet.0 | no-more

show route table inet.3 | no-more

show route table mpls.0 | no-more

show rsvp neighbor | no-more

show rsvp session | no-more

show rsvp session | match DN |no-more

show mpls lsp unidirectional | match Dn | no-more

show ldp neighbor | no-more

show connections | no-more

show log messages | last 200 | no-more

show configuration | display set | count

show configuration | no-more

show configuration |display set | no-more

show services flows brief | no-more

show services sessions | no-more

show services nat pool brief | no-more

show services service-sets cpu-usage | no-more

show services service-sets memory-usage | no-more

show services service-sets summary | no-more

Also, save “request support information” and upload it to NMS.

Below is the example:

jnarveka@CTEFFPRPER02-re0#run request support information |save /var/log/RSI_EFFPER02_12FEB2023.txt

Wrote 32170 lines of output to '/var/log/RSI_EFFPER02_12FEB2023.txt'

Compress the entire /var/log folder as below:

jnarveka@CTEFFPRPER02-re0# run file archive compress source /var/log/* destination /var/tmp/EFFPER02_12FEB2023.tgz

tar: Removing leading '/' from member names

scp the created file to NMS using following example:

jnarveka@nms4:~$ scp effper02.prizmnet:/var/tmp/EFFPER02_12FEB2023.tgz ./

- Backup of active running file system. It will be helpful in case of failure.

request system snapshot

Below command is to validate the snapshot:

show system snapshot

- Check available storage on both routing-engines:

lab@EFFPER02> show system storage

Filesystem              Size       Used      Avail  Capacity   Mounted on

/dev/gpt/junos           19G       6.8G        11G       39%  /.mount

tmpfs                    35G       3.2G        32G        9%  /.mount/tmp

tmpfs                   5.3G       876K       5.3G        0%  /.mount/mfs

lab@EFFPER02> show system storage invoke-on other-routing-engine

re1:

--------------------------------------------------------------------------

Filesystem              Size       Used      Avail  Capacity   Mounted on

/dev/gpt/junos           19G       8.3G       9.4G       47%  /.mount

tmpfs                    35G       3.2G        32G        9%  /.mount/tmp

tmpfs                   5.3G       884K       5.3G        0%  /.mount/mfs

Make sure to have at least 5G-6G free space before proceeding further for upgrade.

In case storage is overutilized, use “request system storage cleanup” command to clean up the storage and type “yes” to clean up in one go or manually delete the unnecessary files using “file delete” command.

request system storage cleanup

We can also check the “show system snapshot” output and delete the older snapshots which are not needed.

- Remove RE-PROTECT filer

- Delete the RE-PROTECT filter on loopback, this is because the Junos files are high in size and we might need to delete/upload the Junos files during upgrade.

#delete interfaces lo0.0 family inet filter

- Once the upgrade is finished, this filter will be re-configured again.

#set interfaces lo0.0 family inet filter input RE-PROTECT

## Junos Software Installation & Upgradation

As per the Junos standard process we can’t upgrade/downgrade to a release which is spaced more than three releases. If you are upgrading from 19.4R3-S10 release to 21.2R3-S6 then you must upgrade the Junos to any 20.4Rx interim release.

We will be using 20.4R3.8 as interim release for the upgrade process. We have put the information related to file name and checksum for interim and Target release in below section.

Link to check list of releases.

https://supportportal.juniper.net/s/article/How-to-check-if-a-Junos-version-after-15-1-is-EEOL

List of EOL/EEOL releases.

https://support.juniper.net/support/eol/software/junos/#15

Table:1

Interim release: 20.4R3.8

File name: junos-install-mx-x86-64-20.4R3.8.tgz

Checksums

MD5 : 7a269aa33dce16fafc7ec4fcb964a975

SHA1 : 83c7362104c6a7315ccca193b635e2eac30d80e0

SHA256 : 6d318ed5eec0b7e81c8641446627112aa4add95d45ee5ca1058d2b55a42e4e3f

SHA512 : e9f0420e102e71f6821c1de4b0a5ebe730b12c8dfdfbe0f4fd3db297ad1de574210e17c94e626eb5097e13780e5cf1fa42063d5dd232916be75d5ce43d0c2887

Table:2

Target release : 21.2R3-S6.11

File name : junos-install-mx-x86-64-21.2R3-S6.11.tgz

Checksums:

MD5 : 8971c215b7e9aa7c9b77e7a98608898b

SHA1 : 929b2fe5a1236a167536da18140a620e9a017360

SHA256: 480b0de456d9f561653b2164cdc858237773e932a62175727b13309ff9e1f278

SHA512: 8d8deb9737d66b787d7dd316dfbb537bcdf76579a0cce5ccfc41b99f47b34ea488e42d68bd70c33f177f785fa7e6c53ffbbec1919a85ae4920ef6d85838de2b8

Steps: Below steps can be used to upgrade the Junos from 19.4R3-S10 release to 20.4R3.8 first. Make sure to use the Junos file/MD5 mentioned in Table:1 (“Interim release”) which is for 20.4R3.8, once device is on 20.4R3.8 release, then proceed with the upgrade to 21.2R3-S6.11 where we need to use Junos file/MD5 mentioned in “Table:2 (Target release).

- Upload both the JUNOS image (20.4R3.8 and 21.2R3-S6.11 to the /var/tmp directory on the router. By default re0 would be active. If we copy software from NMS to MX, then re0 would be having the image not re1. Once we verify the md5 on RE0, then we can directly push the Junos to RE1 from RE0 itself.

- Validate MD5 checksum of the downloaded Junos

@MX-RE0

>file checksum md5 /var/tmp/junos-install-mx-x86-64-20.4R3.8.tgz

compare with MD5 checksum values.

- Upgrade the backup RE1 from RE0

@MX-RE0

lab@EFFPER02> request system software add /var/tmp/junos-install-mx-x86-64-20.4R3.8.tgz re1 no-validate

- This command will take a couple of minutes to complete, it will 1st copy the Junos image into “re1:/var/tmp” directory and then the upgrade process will start.

- no-validate options is required due to OS kernel changes between the releases otherwise it will not allow to add the software and give below error-Same is true with S6 as well)

- Once above process is finished, make sure there are no errors. Then reboot the RE1 from RE0.

- @MX-RE0

request system reboot other-routing-engine

# Wait for RE1 to become back up - user can monitored from RE0 using: “show chassis routing-engine”

- Once RE1 is upgraded to 20.4R3-S8 release, we will upgrade same RE1 to 21.2R3-S6 release.

- Validate MD5 checksum of the downloaded Junos

@MX-RE0

>file checksum md5 /var/tmp/junos-install-mx-x86-64-21.2R3-S6.11.tgz

Compare with MD5 checksum values.

- Upgrade the backup RE1 from RE0 from 20.4R3.8 release to 21.2R3-S6.11 using below command and then reboot the RE.

@MX-RE0

lab@EFFPER02> request system software add /var/tmp/junos-install-mx-x86-64-21.2R3-S6.11.tgz re1 no-validate

request system reboot other-routing-engine

- Once re1 upgraded, it should have both the images in /var/tmp/ directory already. If the file is not available then use below command to copy it manually.

- @MX-RE0

file copy /var/tmp/<filename> re1:/var/tmp/<filename>

- @MX-RE0 ( Do master ship switchover by using below command on RE0)

request chassis routing-engine master switch

- Ensure RE1 become master, all line cards are up on RE1, ensure BGP ,ISIS,LDP neighborship with uplinks are up and stable.

@MX-RE1

show version invoke-on all-routing-engines | no-more

show system uptime | no-more

show ntp associations no-resolve | no-more

show chassis hardware | no-more

show chassis fpc detail | no-more

show chassis alarms | no-more

show system alarms | no-more

show chassis routing-engine | no-more

show chassis environment | no-more

show system resource-monitor fpc | no-more

show krt table | match "kernel-id:  -" |no-more

show system processes extensive | match rpd |no-more

show interface terse | match "up    up" | no-more

show interface terse | no-more

show interfaces descriptions | no-more

show oam ethernet connectivity-fault-management interfaces extensive | no-more

show arp no-resolve | no-more

show vrrp summary | no-more

show lldp neighbors | no-more

show bfd session | no-more

show mpls interface | no-more

show mpls lsp | no-more

show mpls lsp p2mp | no-more

show bgp summary | no-more

show bgp neighbor | no-more

show isis adjacency extensive | no-more

show route summary | no-more

show route table inet.0 | no-more

show route table inet.3 | no-more

show route table mpls.0 | no-more

show rsvp neighbor | no-more

show rsvp session | no-more

show rsvp session | match DN |no-more

show mpls lsp unidirectional | match Dn | no-more

show ldp neighbor | no-more

show connections | no-more

show log messages | last 200 | no-more

show configuration | display set | count

show configuration | no-more

show configuration |display set | no-more

show services flows brief | no-more

show services sessions | no-more

show services nat pool brief | no-more

show services service-sets cpu-usage | no-more

show services service-sets memory-usage | no-more

show services service-sets summary | no-more

Then move to next step

- Verify the md5 checksum as shown earlier for Junos file 20.4R3.8 release and upgrade using below command.

- @MX-RE1 ( Upgrade RE0 from RE1 with Junos 20.4R3.8)

request system software add /var/tmp/<filename> re0 no-validate

(Note this command will take a couple of minutes to complete)

- @MX-RE1 ( Reboot RE0 from RE1)

request system reboot other-routing-engine

- @MX-RE1 ( Upgrade RE0 from RE1 with Junos 21.2R3.S6.11 and then reboot the RE)

request system software add /var/tmp/<filename> re0 no-validate

(Note this command will take a couple of minutes to complete)

@MX-RE1

request system reboot other-routing-engine

- Validate Junos 21.2R3-S6.11 in both the RE’s

@MX-RE-1

show version invoke-on all-routing-engines | no-more

Then move to next step

- @MX-RE1 (Make RE0 Master from RE1)

request chassis routing-engine master switch

# Wait for RE1 to become back up - can be monitored from RE1 using: “show chassis routing-engine”.

- Ensure RE0 become master, all line cards are up on RE0, ensure BGP ,ISIS,LDP neighborship with uplinks are up and stable

@MX-RE0

show version invoke-on all-routing-engines | no-more

show system uptime | no-more

show ntp associations no-resolve | no-more

show chassis hardware | no-more

show chassis fpc detail | no-more

show chassis alarms | no-more

show system alarms | no-more

show chassis routing-engine | no-more

show chassis environment | no-more

show system resource-monitor fpc | no-more

show krt table | match "kernel-id:  -" |no-more

show system processes extensive | match rpd |no-more

show interface terse | match "up    up" | no-more

show interface terse | no-more

show interfaces descriptions | no-more

show oam ethernet connectivity-fault-management interfaces extensive | no-more

show arp no-resolve | no-more

show vrrp summary | no-more

show lldp neighbors | no-more

show bfd session | no-more

show mpls interface | no-more

show mpls lsp | no-more

show mpls lsp p2mp | no-more

show bgp summary | no-more

show bgp neighbor | no-more

show isis adjacency extensive | no-more

show route summary | no-more

show route table inet.0 | no-more

show route table inet.3 | no-more

show route table mpls.0 | no-more

show rsvp neighbor | no-more

show rsvp session | no-more

show rsvp session | match DN |no-more

show mpls lsp unidirectional | match Dn | no-more

show ldp neighbor | no-more

show connections | no-more

show log messages | last 200 | no-more

show configuration | display set | count

show configuration | no-more

show configuration |display set | no-more

show services flows brief | no-more

show services sessions | no-more

show services nat pool brief | no-more

show services service-sets cpu-usage | no-more

show services service-sets memory-usage | no-more

show services service-sets summary | no-more

Take the request support information and save it on NMS server.

Once upgrade is finished we can go ahead with and take the snapshot of the storage.

request system snapshot

Below command is to validate the snapshot:

show system snapshot

There are some changes related to authorization in the Junos above 21.x release where only root and users with Junos OS maintenance permissions can only view the log messages. Below commands needs to be configured after the upgrade to set the permission as per COLT standard.

set system syslog archive world-readable

### Backout

- Rollback to the existing software, we need to follow 21.2R3-S6.11  ---- > 20.4R3.8 ----- > 19..4R3.S10

- Verify the current configuration and outputs from the previously taken backup.

- If there is any issue with the configuration then load the configuration back.