Details

| Author | Jagannath Narvekar |
| --- | --- |
| Owner | Parveen Jindgar |
| File Name & Location |  |

Version History

| Version | Issue Date | Author | Reason for Change |
| --- | --- | --- | --- |
| 1.0 | 13/10/2025 | Jagannath Narvekar | Initial document |

Document Approvals

| Department | Name | Issue | Date of approval |
| --- | --- | --- | --- |
| Packet Network Engineering | Parveen Jindgar |  |  |

Disclosure / Confidentiality

Highly Confidential, not for distribution

Distribution within Colt

Distribution permitted external to Colt

# This document is valid for MX204 devices.

## Prerequisites:

Standard prerequisites for software upgrade:

- Local console accesses of device need to be confirmed prior starting upgrade.

- Back up configuration and capture snapshot of current status of interface, protocols, routing engine, hardware and alarms (if any).

- Telnet/SSH/Console sessions must be captured for entire up-gradation process.

- Check and make sure to have enough storage

## Backup Procedure

Note: All sessions must be logged for commands and command outputs. These logs should be included in the post mortem.

- Save running-configuration to two places (on device locally and NMS)

Juniper machines:

user@DUT> configure private

user@DUT# save filename

user@DUT#run file list --- check if configuration is saved on router

Then use following command to scp the file from router to the NMS server.

nms :~$ scp <DUT device>:/var/tmp/filename ./

Example:

jnarveka@CTEFFPRPER02-re0> configure private

warning: uncommitted changes will be discarded on exit

Entering configuration mode

jnarveka@CTEFFPRPER02-re0# save EFFPER02_10FEB2023

Wrote 40575 lines of configuration to 'EFFPER02_10FEB2023'

jnarveka@CTEFFPRPER02-re0# run file list

var/home/tier3/:

EFFPER02_10FEB2023

jnarveka@nms4:~$ scp effper02.prizmnet:/var/home/tier3/EFFPER02_10FEB2023 ./

Access to this network/system is limited to users authorised by Colt Technology Services Group for its legitimate business purposes. Unauthorised access, use, installation of files or modification of any data is prohibited. Use of this system/network will be audited.

Password:

EFFPER02_10FEB2023   100% |**************************************************************************************************************************************|  1436 KB    00:11

jnarveka@nms4:~$ ls -l

total 2567

-rw-r--r--   1 jnarveka staff    1471111 Feb 10 10:04 EFFPER02_10FEB2023

- For all the routers listed for Upgrade, execute the following show commands and save the information in a text or word document. This information will be used later to check routing.

Make sure to take these outputs pre and post upgrade.

show version | no-more

show system uptime | no-more

show ntp associations no-resolve | no-more

show vmhost version |no-more

show vmhost snapshot |no-more

show chassis hardware | no-more

show chassis fpc detail | no-more

show chassis alarms | no-more

show system alarms | no-more

show chassis routing-engine | no-more

show chassis environment | no-more

show system resource-monitor fpc | no-more

show krt table | match "kernel-id:  -"  |no-more

show system processes extensive | match rpd |no-more

show interface terse | match "up    up" | no-more

show interface terse | no-more

show interfaces descriptions | no-more

show oam ethernet connectivity-fault-management interfaces extensive | no-more

show arp no-resolve | no-more

show vrrp summary | no-more

show lldp neighbors | no-more

show bfd session | no-more

show rsvp neighbor | no-more

show rsvp session | no-more

show route table inet.0 | no-more

show route table inet.3 | no-more

show route table mpls.0 | no-more

show mpls interface | no-more

show mpls lsp | no-more

show mpls lsp p2mp | no-more

show bgp summary | no-more

show bgp neighbor | no-more

show isis adjacency extensive | no-more

show route summary | no-more

show rsvp session | match DN |no-more

show mpls lsp unidirectional | match Dn |no-more

show ldp neighbor | no-more

show connections | no-more

show log messages | last 200 | no-more

show configuration | no-more

show configuration | no-more | display set

show configuration | display set | count

Also, save “request support information” and upload it to NMS.

Below is the example:

jnarveka@CTEFFPRPER02-re0#run request support information |save /var/log/RSI_EFFPER02_12FEB2023.txt

Wrote 32170 lines of output to '/var/log/RSI_EFFPER02_12FEB2023.txt'

Compress the entire /var/log folder as below:

jnarveka@CTEFFPRPER02-re0# run file archive compress source /var/log/* destination /var/tmp/EFFPER02_12FEB2023.tgz

tar: Removing leading '/' from member names

scp the created file to NMS using following example:

jnarveka@nms4:~$ scp effper02.prizmnet:/var/tmp/EFFPER02_12FEB2023.tgz ./

- Backup of active running file system. It will be helpful in case of failure.

- This command will take the snapshot of disk1 onto disk2.

request vmhost snapshot

- Check available storage:

lab@EFFPER01> show system storage

Filesystem              Size       Used      Avail  Capacity   Mounted on

/dev/gpt/junos         10.0G       2.3G       6.9G       25%  /.mount

/dev/gpt/config         952M        96K       876M        0%  /.mount/config

/dev/gpt/var             30G       218M        27G        1%  /.mount/var

tmpfs                    15G        13M        15G        0%  /.mount/tmp

tmpfs                   2.7G       828K       2.7G        0%  /.mount/mfs

Make sure to have at least 5G-6G free space in “var” directory before proceeding further for upgrade.

In case storage is overutilized, use “request system storage cleanup” command to clean up the storage and type “yes” to clean up in one go or manually delete the unnecessary files using “file delete” command.

request system storage cleanup

We can also check the “show system snapshot” output and delete the older snapshots which are not needed.

- Remove the RE-PROTECT filter.

- Delete the RE-PROTECT filter on loopback, this is because the Junos files are high in size and we might need to delete/upload the Junos files during upgrade.

#delete interfaces lo0.0 family inet filter

## Junos Software Installation & Upgradation

As per the Junos standard process we can’t upgrade to a release which is spaced more than three releases. If you are upgrading from 22.2R3-S2.8 release to 23.4R2-S5.6 then you must upgrade the Junos in below order.

22.2R3-S2.8 -> 23.2R2-S4.5 -> 23.4R2-S5.6

Link to check list of releases.

https://supportportal.juniper.net/s/article/How-to-check-if-a-Junos-version-after-15-1-is-EEOL

List of EOL/EEOL releases.

https://support.juniper.net/support/eol/software/junos/#15

Steps:

Upload the JUNOS image to the /tmp directory on the router.

- junos-vmhost-install-mx-x86-64-23.2R2-S4.5.tgz

- junos-vmhost-install-mx-x86-64-23.4R2-S5.6.tgz

- If we add all Junos files at once, it may eat up the disk space. We can upload Junos one by one and delete old Junos files after each upgrade.

- Validate MD5 checksum of the downloaded Junos

>file checksum md5 /var/tmp/<filename>

- compare with MD5 checksum values to validate Junos files based on below.

File name: junos-vmhost-install-mx-x86-64-23.2R2-S4.5.tgz

MD5 : 4abfaccbaca495b699a2cad1bdf5b35a

File name: junos-vmhost-install-mx-x86-64-23.4R2-S5.6.tgz

MD5 : 3e97c983380ac3a1b28e143219c8960a

Install Junos 23.2R2-S4.5 on Disk1.

lab@ESLPER01> request vmhost software add /var/tmp/junos-vmhost-install-mx-x86-64-23.2R2-S4.5.tgz no-validate

(Reboot VMhost)

lab@EFFPER01> request vmhost reboot

# Wait for device to come UP.

Once device come UP with 23.2R2-S4.5, install 23.4R2-S5.6 release using same steps mentioned in step 2 and step 3.

Ensure all services including BGP ,ISIS,LDP neighborship with uplinks are up and stable.

Check following outputs.

show version | no-more

show system uptime | no-more

show ntp associations no-resolve | no-more

show vmhost version |no-more

show vmhost snapshot |no-more

show chassis hardware | no-more

show chassis fpc detail | no-more

show chassis alarms | no-more

show system alarms | no-more

show chassis routing-engine | no-more

show chassis environment | no-more

show system resource-monitor fpc | no-more

show krt table | match "kernel-id:  -"  |no-more

show system processes extensive | match rpd |no-more

show interface terse | match "up    up" | no-more

show interface terse | no-more

show interfaces descriptions | no-more

show oam ethernet connectivity-fault-management interfaces extensive | no-more

show arp no-resolve | no-more

show vrrp summary | no-more

show lldp neighbors | no-more

show bfd session | no-more

show rsvp neighbor | no-more

show rsvp session | no-more

show route table inet.0 | no-more

show route table inet.3 | no-more

show route table mpls.0 | no-more

show mpls interface | no-more

show mpls lsp | no-more

show mpls lsp p2mp | no-more

show bgp summary | no-more

show bgp neighbor | no-more

show isis adjacency extensive | no-more

show route summary | no-more

show rsvp session | match DN |no-more

show mpls lsp unidirectional | match Dn |no-more

show ldp neighbor | no-more

show connections | no-more

show log messages | last 200 | no-more

show configuration | no-more

show configuration | no-more | display set

show configuration | display set | count

Also, save “request support information” and upload it to NMS.

## Taking VMhost snapshot:

There are two disks in MX204, Disk1 is considered primary and Disk2 is backup.

There are two sets of software images on each disk.

Set-p and Set-b. Router boots from one of the set, it could be either set-p or set-b. It can be verified with command “show vmhost version” which set is device currently booted from.

For example:

In below output, we can see that the device is currently booted from partition “sda4” which is Primary “Disk1”.

Also, “Current boot set:b” shows that the device is booted from “set-b”.

We can also see that the “set-p” has older release which is “18.2R3-S5.3”.

This other set (set-p in below example) will be used when we perform Junos vmhost upgrade or Junos vmhost rollback” option.

lab@EFFPER01> show vmhost version

Current root details,           Device sda, Label: jrootb_P, Partition: sda4

Current boot disk: Primary

Current root set: b

UEFI    Version: CBEP_P_SUM1_00.15.01

Primary Disk, Upgrade Time: Thu Jan 26 16:57:56 UTC 2023

Version: set p

VMHost Version: 3.1836

VMHost Root: vmhost-x86_64-18.2R3-S4-20200419_2309_builder

VMHost Core: vmhost-core-x86-64-18.2R3-S5.3

kernel: 3.10.100-ovp-rt110-WR6.0.0.31_preempt-rt

Junos Disk: junos-install-mx-x86-64-18.2R3-S5.3

Version: set b

VMHost Version: 5.2245

VMHost Root: vmhost-x86_64-19.4R3-S7-20211221_2131_builder

VMHost Core: vmhost-core-x86-64-19.4R3-S10.3

kernel: 4.8.28-rt10-WR9.0.0.20_ovp

Junos Disk: junos-install-mx-x86-64-19.4R3-S10.3

In case of primary disk1 failure, device will boot from disk2. When we confirm that all the services are working fine then we need to take a snapshot of Disk1 on Disk2.

To take snapshot use following command.

request vmhost snapshot

Verify if the snapshot is taken correctly.

Below command should show the time when the snapshot was taken and it should mention “Secondary Disk”.

lab@CTEFFPRPER01> show vmhost version

Current root details,                   Device sda, Label: jrootp_P, Partition: sda3,

Current boot disk: Primary

Current root set: p

UEFI    Version: CBEP_P_SUM1_00.15.01

Primary Disk, Upgrade Time: Thu Jul 31 11:55:32 UTC 2025

Version: set p

VMHost Version: 7.2955

VMHost Root: vmhost-x86_64-23.4R2-S5-20250515_2123_builder

VMHost Core: vmhost-core-x86-64-23.4R2-S5.6

kernel: 5.2.60-rt15-LTS19

Junos Disk: junos-install-mx-x86-64-23.4R2-S5.6

Version: set b

VMHost Version: 7.2753

VMHost Root: vmhost-x86_64-23.2R2-S3-20241205_0251_builder

VMHost Core: vmhost-core-x86-64-23.2R2-S4.5

kernel: 5.2.60-rt15-LTS19

Junos Disk: junos-install-mx-x86-64-23.2R2-S4.5

In case of failure of the primary disk, you can boot from the image available in the backup disk and then recover the primary disk with the snapshot created using the recovery option.

- Once the upgrade is finished, RE-PROTECT filter will be re-configured again.

#set interfaces lo0.0 family inet filter input RE-PROTECT

### Backout

- Rollback to the existing software, same steps we can take to rollback with old Junos from which we upgraded.

- Verify the current configuration and outputs from the previously taken backup.

- If there is any issue with the configuration then load the configuration back.